Lower Mokau – Quarterly River Monitoring

Sample Collection Day: 13th November 2023

Overall, water quality at Mokau River, above the Wairere Dam was good on 13th November 2023.

E. coli was low (190). Nitrate and Ammonia concentrations were well below toxicity levels (Nitrate 0.37 mg/L; Ammonia 0.02 mg/L). Dissolved inorganic nitrogen was low (0.39 mg/L). Dissolved reactive phosphorus was exceptionally low (0.006 mg/L). Water clarity was poor (1.03 m) relative to the national bottom line (1.34 m).

Mokau River – Summary of water quality collected at 16 sites across the Mokau River catchment November 2023

Nutrient concentrations (nitrate, ammonia and dissolved reactive phosphorus) were low at most sites. Key contaminants were E. coli and water clarity.

E. coli: 56% of all sites had low concentrations (≤230), 25% had slightly elevated concentrations (between 280 - 530) and 19% had elevated concentrations (> 730). Nitrate and Ammonia: concentrations at 100% of sites were below toxicity levels (Nitrate ≤ 0.62 mg/L; Ammonia ≤ 0.02). Only one site had Dissolved Inorganic Nitrogen (DIN) concentrations above 0.5 mg/L. Ecological impacts, including problematic growth of algae and/or aquatic plants and loss of sensitive aquatic species are likely to occur when DIN regularly exceeds 0.5 mg/L. Dissolved reactive phosphorus: 94% of sites had low concentrations (≤ 0.009 mg/L) and one site (16%) had slightly elevated concentrations (0.011 mg/L).Water clarity: 44% of sites had good water clarity (A or B band), two sites (13%) had reduced water clarity (C band) and 44% of sites had poor water clarity (D band). Bands for each site relate to the national bottom line (NBL) for water clarity and are dependent on landscape characteristics including geology, climate and elevation. The NBL for Mokau River’s monitoring sites are either 1.34 m or 0.61 m, dependant the local landscape characteristics.

Data source: Waikato Regional Council envirohub website for environmental data.