Lower Mokau – Quarterly River Monitoring

Sample Collection Day: 28th February 2024

Overall, water quality was Fair above the Wairere Dam on Mokau River.

E. coli was slightly elevated (320) but fell within recommended health limits for swimming (540). Nitrate concentrations were well below ecological toxicity impacts (2.4 mg/L). Ammonia concentrations were also low, falling below ecological toxicity impacts (0.24 mg/L) Dissolved inorganic nitrogen was low (0.39 mg/L). Dissolved reactive phosphorus was exceptionally low (0.006 mg/L). Water clarity was poor (1.03 m) relative to the national bottom line (1.34 m).

Mokau River – Summary of water quality collected at 15 sites across the Mokau River catchment February 2024

Nutrient concentrations (nitrate, ammonia and dissolved reactive phosphorus) were low at most sites. Key contaminants were E. coli and suspended sediment.

E. coli: 60% of all sites had low concentrations (≤260) and 40% had slightly elevated concentrations (between 300 - 450). Nitrate and Ammonia: 100% of sites were well below toxicity levels. Nitrate range (0.04 – 0.53 mg/L); Ammonia range (<0.005 - 0.03). Dissolved inorganic nitrogen: 87% of sites had low concentrations (≤0.4 mg/L) and 13% (2 sites) fell above the ecological impact threshold of 0.5 mg/L with the highest concentration being 0.54 mg/L. Dissolved reactive phosphorus: 100% of sites had low concentrations (between <0.002 - 0.01 mg/L). Water clarity: 53% of sites had good water clarity (A or B band), 13% (2 sites) had reduced water clarity (C band) and 33% of sites had poor water clarity (D band). Bands for each site relate to the national bottom line (NBL) for water clarity and are dependent on landscape characteristics including geology, climate and elevation. The NBL for Mokau River’s monitoring sites are either 1.34 m or 0.61 m, dependant the local landscape characteristics.