Lower Mokau – Quarterly River Monitoring

Sample Collection Day: 14th May 2024

Overall, water quality was Good above the Wairere Dam on Mokau River, only water clarity was slightly slow.

E. coli was very low (110) and well within recommended health levels for swimming (540). Nitrate concentrations were well below ecological toxicity impacts (2.4 mg/L). Ammonia concentrations were exceptionally low, falling well below ecological toxicity impacts (0.24 mg/L) Dissolved inorganic nitrogen was low (0.43 mg/L). Dissolved reactive phosphorus was exceptionally low (0.004 mg/L). Water clarity was fair (1.52 m), relative to the national bottom line (1.34 m).

Mokau River – Summary of water quality collected at 15 sites across the Mokau River catchment May 2024

E. coli and Nutrients (nitrate, ammonia and dissolved reactive phosphorus) were low at most sites. The key contaminant was suspended sediment.

E. coli: 93% of all sites had low concentrations (≤240) and 7% (1 site) had elevated concentrations (510). Nitrate and Ammonia: 100% of sites were well below toxicity levels. Nitrate range (0.15 – 0.79 mg/L); Ammonia range (<0.005 - 0.02). Dissolved inorganic nitrogen: 73% of sites had low concentrations (≤0.44 mg/L) and 27% (4 sites) fell above the ecological impact threshold of 0.5 mg/L with the highest concentration being 0.79 mg/L. Dissolved reactive phosphorus: 100% of sites had low concentrations (between <0.002 - 0.008 mg/L). Water clarity: 60% of sites had good water clarity (A or B band), 20% (3 sites) had reduced water clarity (C band) and the remaining 20% had poor water clarity (D band). Bands for each site relate to the national bottom line (NBL) for water clarity and are dependent on landscape characteristics including geology, climate and elevation. The NBL for Mokau River’s monitoring sites are either 1.34 m or 0.61 m, dependant the local landscape characteristics.