Awakino River & Tasman - Quarterly River Monitoring

Sample Collection Day: 12th June 2024

Overall, water quality at Mangaorongo stream was Good, except for Dissolved Inorganic Nitrogen, which was slightly elevated.

E. coli concentrations were low (210), falling within recommending health levels for swimming (540). Nitrate and Ammonia concentrations were well below levels which can cause ecological toxicity impacts (Nitrate 2.4 mg/L; Ammonia 0.24 mg/L). Dissolved inorganic nitrogen was slightly elevated (0.59 mg/L), potentially impacting ecological health by promoting excessive growth of algae, aquatic plants and impacting sensitive aquatic species. Dissolved reactive phosphorus was low (≥ 0.01 mg/L). Water clarity was good (0.91 m), relative to the national bottom line (0.61 m), indicating relatively low concentrations of suspended sediment.

Awakino River - Rauroa Farm Bridge - River level

Data source: Waikato Regional Council envirohub website for environmental data.