Lower Mokau – Quarterly River Monitoring

Sample Collection Day: 19th November 2024

Water quality was Fair. E. coli and suspended sediment were elevated, while nutrient indicators were low.

E. coli was elevated (700) falling above the recommended health levels for swimming (540). Nitrate concentrations were low (0.44 mg/L), falling well below levels likely to cause ecological toxicity impacts (2.4 mg/L). Ammonia concentrations were also low, falling well below ecological toxicity impacts (0.24 mg/L) Dissolved inorganic nitrogen (DIN) were low (0.46 mg/L), falling just below levels considered to potentially impacting ecological health. DIN concentrations greater than 0.5 mg/L can cause ecological impacts like excessive growth of algae and aquatic plants, and loss of sensitive species. Dissolved reactive phosphorus was very low (0.006 mg/L). Water clarity was very poor (0.64 m), relative to the national bottom line (1.34 m).

Mokau River – Summary of water quality collected at 15 sites across the Mokau River catchment on 19th November 2024

E. coli and suspended sediment were elevated across most sites and a third of all sites had elevated concentrations of dissolved inorganic nitrogen. While dissolved reactive phosphorus (DRP) was generally low, very high DRP concentrations were recorded at several locations.

E. coli: 73% of sites had elevated concentrations (between 610 and 1,500), 20% (3 sites) had slightly elevated concentrations (between 370 and 470), while only 7% (1 site) had a low concentration (≤ 110). Nitrate and Ammonia: 100% of sites were well below toxicity levels. Nitrate range (0.13 – 0.80 mg/L); Ammonia range (<0.005 - 0.05). Dissolved inorganic nitrogen (DIN): 67% of sites had low concentrations (≤ 0.46 mg/L) and 33% were slightly elevating, falling above the ecological impact threshold of 0.5 mg/L (between 0.54 and 0.81 mg/L). The highest DIN concentration recorded across the Mokau River catchment was 0.81 mg/L. Dissolved reactive phosphorus: 73% of sites had low concentrations (between <0.002 to 0.010 mg/L), 7% (1 site) was slightly elevated (0.017 mg/L) and 20% (3 sites) had very high concentrations (between 0.019 and 0.065 mg/L). Water clarity: 13% (2 sites) had good water clarity (A or B band), 13% (2 sites) had reduced water clarity (C band) and the remaining 73% had poor water clarity (D band). Bands for each site relate to the national bottom line (NBL) for water clarity and are dependent on landscape characteristics including geology, climate and elevation. The NBL for Mokau River’s monitoring sites are either 1.34 m or 0.61 m, dependant on the local landscape characteristics.

River Level and Flow Rate – Mokau River, Totoro Road

The below charts present continuous data collected by the Waikato Regional Council for Mokau River between 1st January and 30th November 2024. River Level and Flow Rate on the day of sampling (19-November) are highlighted on the second chart.

Data source: Waikato Regional Council envirohub website for environmental data.